CUSTOMER REQUIREMENT SPECIFICATION (CRS)

Project Information

Project Name: JE EHQ00012 08642 – 12V BSD Actuator Controller POC
Customer ID: CUS-JE-ACT-2026
Document ID: CUD-JE-ACT-CRS-001
System Requirement Specification ID: SyRS-JE-ACT-STM32F446-001
MCU Platform: STM32F446RE
Revision: 1.0
Date: 28-Feb-2026
Confidentiality: Customer Use Only

TABLE OF CONTENTS

- Introduction

- Applicable Standards

- System Overview

- ASPICE Process Alignment

- Functional Requirements

- Electrical Requirements

- Microcontroller Requirements – STM32F446RE

- Software Requirements

- Control Logic Specification

- Communication Requirements (UART)

- Protection & Diagnostics

- Environmental Requirements

- Risk & Assumptions

- Failure Mode and Effects Analysis (FMEA)

- Requirement Traceability Matrix (RTM)

- Validation & Acceptance Criteria

- Deliverables

1. INTRODUCTION

1.1 Purpose

This document defines the system, hardware, and software requirements for development of a Proof of Concept (POC) controller for the JE EHQ00012 08642 12V BSD actuator used in automotive applications.

The system shall implement ADC-based closed-loop control using potentiometer feedback and shall provide UART-based control and diagnostics.

2. APPLICABLE STANDARDS

The system design shall consider compliance with the following standards:

- ISO 16750 – Road Vehicles Environmental Conditions

- ISO 7637 – Electrical Transient Immunity

- IEC 61000-4 – EMC Immunity

- MISRA-C – Software Coding Guidelines (Recommended)

3. SYSTEM OVERVIEW

3.1 Actuator Details

- Model: JE EHQ00012 08642

- Supply Voltage: 12V DC

- Feedback Type: Analog Potentiometer

3.2 System Objectives

The controller shall:

- Read the ADC value from the actuator feedback potentiometer.

- Control the actuator based on the measured ADC value.

- Stop the actuator at a defined shipping position represented by a configured ADC threshold.

- Allow actuator ON and OFF control through UART communication.

4. ASPICE PROCESS ALIGNMENT

| ASPICE Process | Application |
| --- | --- |
| SYS.2 | System Requirements Definition |
| SYS.3 | System Architectural Design |
| SWE.1 | Software Requirements Analysis |
| SWE.2 | Software Architectural Design |
| SWE.4 | Software Unit Design |
| SUP.1 | Configuration Management |

5. FUNCTIONAL REQUIREMENTS

SyRS-FUNC-001: ADC Acquisition

The system shall continuously acquire actuator feedback using the internal ADC.

Requirements:

- ADC resolution shall be minimum 12-bit.

- ADC sampling frequency shall be ≥ 1 kHz.

- ADC value shall be treated as the primary feedback variable.

SyRS-FUNC-002: ADC-Based Flap Actuation Control

The system shall control flap actuation exclusively based on the measured ADC feedback value from the actuator potentiometer.

Requirements:

- Flap actuation shall operate within an ADC range of 500 to 2600 counts.

- Control decisions shall be derived solely from the real-time ADC feedback signal.

- Open-loop control shall not be permitted under any operating condition.

- The actuator shall move the flap only within the defined ADC window (500 ≤ ADC ≤ 2600).

- The actuator drive output shall be disabled immediately if:

  - ADC value falls below 500, or

  - ADC value exceeds 2600.

- The actuator shall stop when the configured target ADC value within the 500–2600 range is reached.

SyRS-FUNC-003: Overvoltage Protection

The system shall monitor the external power supply voltage using an ADC channel through a resistive voltage divider network.

Requirements:

- The external supply voltage shall be scaled using a voltage divider and measured via an internal ADC channel.

- The measured ADC value shall be continuously monitored to determine the actual supply voltage.

- A configurable threshold value (Overvoltage_Threshold) shall define the maximum permissible supply voltage.

- When the measured supply voltage equals or exceeds the configured Overvoltage_Threshold:

  - Actuator drive output shall be disabled immediately.

  - Further actuator operation shall be blocked.

  - System state shall transition to FAULT.

  - UART diagnostic message “OVERVOLTAGE_FAULT_ACTIVE” shall be transmitted.

- The system shall remain in FAULT state until:

  - Supply voltage returns within acceptable limits, and

  - System reset or authorized UART command is received.

- The overvoltage detection mechanism shall operate continuously during all active system states.

SyRS-FUNC-004: UART-Based Actuator Control

The system shall allow actuator control via UART commands.

Minimum supported commands:

- ACTUATOR_ON

- ACTUATOR_OFF

System Behavior:

- Upon receiving ACTUATOR_ON, actuator operation shall begin according to control logic.

- Upon receiving ACTUATOR_OFF, actuator output shall be disabled immediately.

- Invalid commands shall be ignored and logged.

6. ELECTRICAL REQUIREMENTS

SyRS-ELEC-001: Supply Requirements

- Nominal Supply Voltage: 12V

- Operating Range: 9V – 16V

- Reverse polarity protection shall be implemented.

- Transient voltage suppression shall be implemented.

SyRS-ELEC-002: ADC Interface

- ADC resolution: Minimum 12-bit

- Reference Voltage: 3.3V

- Sampling frequency: ≥ 1 kHz

- ADC input shall include overvoltage protection.

7. MICROCONTROLLER REQUIREMENTS – STM32F446RE

7.1 Device Specification

- Core: ARM Cortex-M4 with FPU

- Maximum Frequency: 120 MHz

- Flash Memory: 512 KB

- SRAM: 128 KB

7.2 Required Peripherals

The MCU shall provide:

- 12-bit ADC – PA1

- GPIO Actuator Control Pin – PA5

- Advanced PWM Timers

- UART (≥1 channel)

- DMA support

- Independent Watchdog

System clock shall be configured to ensure stable control loop operation.

8. SOFTWARE REQUIREMENTS

8.1 Software Architecture

The software shall follow a layered architecture:

- HAL Layer

- Driver Layer

- Middleware Layer

- Application Layer

8.2 Operating States

The system shall implement the following states:

- INIT

- IDLE

- ACTIVE

- SHIPPING_LOCK

- FAULT

8.3 Watchdog Supervision

An independent watchdog shall be implemented.
The system shall reset automatically in case of software malfunction.

9. CONTROL LOGIC SPECIFICATION

Closed-loop control shall be implemented using ADC feedback only.

Requirements:

- Control loop frequency shall be ≥ 1 kHz.

- Actuator shall stop at configured ADC thresholds.

- Control logic shall prevent oscillation or unstable behavior.

- No physical position or angle conversion shall be implemented.

10. COMMUNICATION REQUIREMENTS (UART)

SyRS-COMM-001: Monitoring & Control

The system shall periodically transmit:

- Raw ADC value

- System state

- Shipping lock status

- Fault flags

UART Configuration:

- Baud Rate: 115200

- Format: 8N1

- Transmission interval ≤ 100 ms

Example Frame:

<STX>ADC=2048,STATE=ACTIVE,SHIP_LOCK=0,FAULT=0<ETX>

11. PROTECTION & DIAGNOSTICS

The system shall implement:

- Overcurrent shutdown

- Thermal shutdown (MCU internal temperature monitoring)

- Stall detection

- Short-circuit protection

- Undervoltage detection (<9V)

- Watchdog supervision

Any fault condition shall transition the system to FAULT state.

12. ENVIRONMENTAL REQUIREMENTS

- Operating Temperature: -40°C to +85°C

- Storage Temperature: -40°C to +105°C

- Humidity: 95% RH non-condensing

- Automotive-grade PCB design recommended

13. RISK & ASSUMPTIONS

13.1 Assumptions

- Potentiometer feedback is linear within operating range.

- 12V supply is automotive compliant.

13.2 Risks and Mitigation

| Risk ID | Description | Mitigation |
| --- | --- | --- |
| R-01 | ADC noise affects control | Digital filtering |
| R-02 | Mechanical wear | Periodic recalibration |
| R-03 | Motor stall | Current monitoring shutdown |
| R-04 | Software crash | Watchdog reset |

14. FAILURE MODE AND EFFECTS ANALYSIS (FMEA)

| Item | Failure Mode | Effect | Severity | Detection | Mitigation |
| --- | --- | --- | --- | --- | --- |
| ADC | Incorrect reading | Incorrect control | High | Range check | Filtering |
| MCU | Crash | Loss of control | High | Watchdog | Auto reset |
| POT | Open circuit | Loss of feedback | Medium | ADC out-of-range | FAULT state |
| Motor | Stall | Overcurrent | High | Current sense | Shutdown |

15. REQUIREMENT TRACEABILITY MATRIX (RTM)

| Requirement ID | Description | HW Allocation | SW Allocation | Test Case |
| --- | --- | --- | --- | --- |
| SyRS-FUNC-001 | ADC acquisition | ADC | ADC Driver | TC-01 |
| SyRS-FUNC-002 | ADC-based control | GPIO | Control Logic | TC-02 |
| SyRS-FUNC-003 | Shipping lock | ADC + GPIO | State Machine | TC-03 |
| SyRS-COMM-001 | UART control | UART | Communication Layer | TC-04 |

16. VALIDATION & ACCEPTANCE CRITERIA

The POC shall demonstrate:

- Continuous ADC acquisition.

- Actuator control based on ADC feedback.

- Hard stop at configured Shipping_ADC_Value.

- Successful ON/OFF control via UART.

- Correct fault detection and system state transition.

17. DELIVERABLES

Supplier shall provide:

- System Architecture Diagram

- Schematic

- PCB Layout

- Firmware Source Code

- Calibration Procedure

- Test Report

- Demonstration Video